Lab Manual for Computer Communication and Networking

Lab No. 09

Open Shortest Path First(OSPF)

BAHRIA UNIVERSITY KARACHI CAMPUS

Department of Software Engineering

COMPUTER COMMUNICATION & NETWORKING

LAB EXPERIMENT # 09

# Dynamic Routing OSPF

OBJECTIVE: -

- This lab assignment helps in understanding how dynamic routing using Routing Information Protocol (OSPF) can be configured on routers.

THEORY: -

OSPF Properties

Open Standard

AD=110

Metric = Bandwidth

Makes 3-Table (Routing, Topology, Neighbor)

Hello packets = 10 sec

Dead Interval = 40 sec

Support Multicast

Algorithm = Dijkstra’s

NETWORK TOPOLOGY: -

PROCEDURE AND OBSERVATION: -

Step01: Configuring OSPF routing on router 1

Router1(config)#interface serial 0/0/0	  (Configuring serial 0 port)

Router1(config-if)#ip address 15.0.0.1 255.0.0.0

Router1(config-if)#Clock rate 64000

Router1(config-if)#no shut

Router1(config-if)#exit

Router1(config)#interface fa 0/0 	  (Configuring fastethernet 0 port)

Router1(config-if)#ip address 10.0.0.3 255.0.0.0

Router1(config-if)#no shut

Router1(config-if)#exit

Router1(config)#router OSPF	 1		  (Configuring OSPF on router 1)

Router1(config)#network 10.0.0.0 0.255.255.255 area 0

Router1(config)#network 15.0.0.0	0.255.255.255 area 0

Router1# show ip route

Step 02: Configuring static routing on router 2

Router2(config)#interface serial 0/0/0	  (Configuring serial 0 port)

Router2(config-if)#ip address 15.0.0.5 255.0.0.0

Router2(config-if)#Clock rate 64000

Router2(config-if)#no shut

Router2(config-if)#exit

Router2(config)#interface fa 0/0 	  (Configuring fastethernet 0 port)

Router2(config-if)#ip address 20.0.0.3 255.0.0.0

Router2(config-if)#no shut

Router2(config-if)#exit

Router2(config)#router OSPF	 1		  (Configuring OSPF on router 2)

Router2(config)#network 20.0.0.0 0.255.255.255 area 0

Router2(config)#network 15.0.0.0	0.255.255.255 area 0

Router2# show ip route

Step03: Verify the route by pinging from Router 1 to Router 2

Router2# ping 20.0.0.2				or

Router1# ping 10.0.0.2

Step04: Verify the route by pinging from PC 1 to PC3

C:\> ping 10.0.0.1 	(from PC 1)		or

C:\> ping 20.0.0.2	(from PC 3)

QUESTIONS: -

- Configure OSPF on the following network and show all necessary configuration steps for each router.

Solution

Router 0

Router 1

Setting OSPF on router 0

Setting OSPF on router 1

- Configure OSPF on the following network and show all necessary configuration steps for each router.

Solution

Router 0

Setting OSPF on router 0

Router 1

Setting OSPF on router 1

Router 2

Setting OSPF on router 2

Ping

TIME BOXING:

| Activity Name | Activity Time | Total Time |
| --- | --- | --- |
| Instruments Allocation + Setting up Lab | 10 mints | 10 mints |
| Walk through Theory & Tasks (Lecture) | 60 mints | 60 mints |
| Implementation & Practice time | 90 mints | 80 mints |
| Evaluation Time | 20 mints | 20 mints |
|  | Total Duration | 180 mints |

Teacher Signature: 		________________________

Student Registration No:	________________________